Feet MSL

1.  What is the drop altitude in feet MSL for a UH-60 dropping door bundles during the day if the highest point on the drop zone is 113 feet above sea level?

450 MSL

2.  What is the drop altitude in feet MSL for a UH-1 dropping door bundles at night if the highest point on the drop zone is 248 feet above sea level?

750 MSL

3.  What is the drop altitude in feet MSL for a CH-47 dropping jumpers at night if the highest point on the drop zone is 303 feet above sea level?

1850 MSL

4.  What is the minimum drop altitude in feet MSL for a CH-47 dropping jumpers at night if the highest point on the drop zone is 118 feet above sea level?

1400 MSL

5.  What is the minimum drop altitude in feet MSL for a C130 dropping door bundles using the T-10 cargo parachute at night if the highest point on the drop zone is 287 feet above sea level?

700 MSL

6.  What is the minimum drop altitude in feet MSL for a C130 dropping door bundles using the G14 parachute at night if the highest point on the drop zone is 96 feet above sea level?

400 MSL

7.  What is the minimum drop altitude in feet MSL for a C17 dropping door bundles using the T-10 cargo parachute at night if the highest point on the drop zone is 442 feet above sea level?

750 MSL

8.  What is the minimum drop altitude in feet MSL for a C17 dropping door bundles using the G14 parachute at night if the highest point on the drop zone is 134 feet above sea level?

450 MSL

9.  What is the drop altitude in feet MSL for a C130 dropping door bundles using the T-10 cargo parachute at night if the highest point on the drop zone is 289 feet above sea level?

1300 MSL

10.  What is the minimum drop altitude in feet MSL for a C130 dropping CDS using the G14 parachute at night if the highest point on the drop zone is 231 feet above sea level?

650 MSL

11.  What is the minimum drop altitude in feet MSL for a C17 dropping CDS using the G14 parachute at night if the highest point on the drop zone is 18 feet above sea level?

350 MSL

12.  What is the drop altitude in feet MSL for a C130 dropping CDS using the G14 parachute at night if the highest point on the drop zone is 118 feet above sea level?

750 MSL

13.  What is the minimum drop altitude in feet MSL for a C130 dropping heavy equipment using the G12E parachute at night if the highest point on the drop zone is 167 feet above sea level?

750 MSL

14.  What is the minimum drop altitude in feet MSL for a C17 dropping heavy equipment using the 5000 lbs parachute release system at night if the highest point on the drop zone is 112 feet above sea level?

1150 MSL

15.  What is the drop altitude in feet MSL for a C130 dropping heavy equipment using the G12E parachute at night if the highest point on the drop zone is 332 feet above sea level?

1450 MSL

16.  What is the drop altitude in feet MSL for a C130 dropping jumpers conducting tactical training at night if the highest point on the drop zone is 67 feet above sea level?

900 MSL

17.  What is the drop altitude in feet MSL for a C130 dropping jumpers at night if the highest point on the drop zone is 568 feet above sea level?

1600 MSL

18.  What is the drop altitude in feet MSL for a C130 dropping SATB-Ps from 500 feet AGL if the highest point on the drop zone is 678 feet above sea level?

1200 MSL

19.  What is the drop altitude in feet MSL for a C130 dropping basic airborne students at night if the highest point on the drop zone is 156 feet above sea level?

1450 MSL

20.  What is the drop altitude in feet MSL for a C17 dropping jumpers and heavy equipment at night if the highest point on the drop zone is 101 feet above sea level?

1250 MSL

21.  What is the drop altitude in feet MSL for a C17 dropping jumpers and CDS at night if the highest point on the drop zone is 322 feet above sea level?

1350 MSL

22.  What is the drop altitude in feet MSL for a C130 dropping CDS and heavy equipment at night if the highest point on the drop zone is 543 feet above sea level?

1650 MSL

23.  What is the drop altitude in feet MSL for a C17 dropping door bundles and CDS at night if the highest point on the drop zone is 124 feet above sea level?

1150 MSL

24.  What is the drop altitude in feet MSL for a C130 dropping door bundles and heavy equipment using the 5000 lbs release system at night if the highest point on the drop zone is 234 feet above sea level?

1350 MSL

25.  What is the drop altitude in feet MSL for a C130 dropping jumpers and door bundles at night if the highest point on the drop zone is 513 feet above sea level?

1550 MSL

CARP DROP ZONE SIZES

- What size CARP drop zone is required for 3 C130Js dropping 64 jumpers and 2 door bundles each mass exit at night using GPS?

|  | W | L |
| --- | --- | --- |
| 1 | 600 | 600 |
| N | 100 | 100 |
| A | 0 | 0 |
| N | 0 | 0 |
| A | 0 | 2475 |
| S | 0 | 0 |
|  | 700 | 3175 |

- What size CARP drop zone is required for 1 C130H dropping 5 CDS bundles on a single roller system at night using GPS?

|  | W | L |
| --- | --- | --- |
| 1 | 400 | 700 |
| N | 100 | 100 |
| A | 0 | 0 |
| N | 0 | 0 |
| A | 0 | 0 |
| S | 0 | 0 |
|  | 500 | 800 |

- What size CARP drop zone is required for 2 C130Js dropping 5 heavy equipment platforms each at night?

|  | W | L |
| --- | --- | --- |
| 1 | 600 | 1000 |
| N | 100 | 100 |
| A | 0 | 0 |
| N | 0 | 0 |
| A | 0 | 1600 |
| S | 0 | 0 |
|  | 700 | 2700 |

- What size CARP drop zone is required for 2 C130Js dropping 7 CDS bundles on a double roller system each flying staggered trail using SKE?

|  | W | L |
| --- | --- | --- |
| 1 | 400 | 550 |
| N | 0 | 0 |
| A | 0 | 0 |
| N | 0 | 0 |
| A | 0 | 0 |
| S | 400 | 0 |
|  | 800 | 550 |

- What size CARP drop zone is required for 2 C130Hs dropping 3 heavy equipment platforms each using SKE?

|  | W | L |
| --- | --- | --- |
| 1 | 600 | 1000 |
| N | 0 | 0 |
| A | 0 | 0 |
| N | 0 | 0 |
| A | 0 | 800 |
| S | 400 | 0 |
|  | 1000 | 1800 |

- What size CARP drop zone is required for 3 C130Js dropping 76 jumpers and 1 door bundles each mass exit at night from 1700 feet AGL flying NIT right using SKE and GPS?

|  | W | L |
| --- | --- | --- |
| 1 | 600 | 600 |
| N | 100 | 100 |
| A | 210 | 210 |
| N | 0 | 0 |
| A | 0 | 2850 |
| S | 400 | 0 |
|  | 1310 | 3760 |

- What size CARP drop zone is required for 3 C130Js dropping 3 heavy equipment platforms each at night from 1300 feet AGL using GPS?

|  | W | L |
| --- | --- | --- |
| 1 | 600 | 1000 |
| N | 100 | 100 |
| A | 60 | 60 |
| N | 0 | 0 |
| A | 0 | 800 |
| S | 0 | 0 |
|  | 760 | 1960 |

- What size CARP drop zone is required for 3 C130Hs dropping 4 heavy equipment platforms each at night from 1850 feet AGL flying NIT using SKE?

|  | W | L |
| --- | --- | --- |
| 1 | 600 | 1000 |
| N | 100 | 100 |
| A | 210 | 210 |
| N | 0 | 0 |
| A | 0 | 1200 |
| S | 400 | 0 |
|  | 1310 | 2510 |

- What size CARP drop zone is required for 1 C130J dropping 24 jumpers and 3 door bundles at night from 1400 feet AGL using GPS?

|  | W | L |
| --- | --- | --- |
| 1 | 600 | 600 |
| N | 0 | 100 |
| A | 120 | 120 |
| N | 0 | 0 |
| A | 0 | 1950 |
| S | 0 | 0 |
|  | 720 | 2770 |

- What size CARP drop zone is required for 2 C130Js dropping 53 jumpers and 3 door bundles each mass exit at night from 1800 feet AGL flying staggered trail using SKE and GPS?

|  | W | L |
| --- | --- | --- |
| 1 | 600 | 600 |
| N | 100 | 100 |
| A | 240 | 240 |
| N | 0 | 0 |
| A | 0 | 2100 |
| S | 400 | 0 |
|  | 1340 | 3040 |

- What size CARP drop zone is required for 2 C130Js if the lead aircraft is dropping 5 heavy equipment platforms and the second aircraft is dropping 64 jumpers and 2 door bundles mass exit at night using GPS?

|  | W | L |  | W | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 600 | 1000 | 1 | 600 | 600 |
| N | 100 | 100 | N | 100 | 100 |
| A | 0 | 0 | A | 30 | 30 |
| N | 0 | 0 | N | 0 | 0 |
| A | 0 | 1600 | A | 0 | 2475 |
| S | 0 | 0 | S | 0 | 0 |
|  | 700 | 2700 |  | 730 | 3205 |

- What size CARP drop zone is required for 2 C130Hs if the lead aircraft is dropping 5 CDS bundles on a single roller system and the second aircraft is dropping 15 jumpers and 2 door bundles at night flying NIT?

|  | W | L |  | W | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 400 | 700 | 1 | 600 | 600 |
| N | 100 | 100 | N | 100 | 100 |
| A | 160 | 160 | A | 0 | 0 |
| N | 100 | 0 | N | 100 | 0 |
| A | 0 | 0 | A | 0 | 1200 |
| S | 0 | 0 | S | 0 | 0 |
|  | 760 | 960 |  | 800 | 1900 |

- What size CARP drop zone is required for 2 C130Hs if the lead aircraft is dropping 2 heavy equipment platforms and the second aircraft is dropping 4 CDS bundles on a single roller system at night flying staggered trail?

|  | W | L |  | W | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 600 | 1000 | 1 | 400 | 550 |
| N | 100 | 100 | N | 100 | 100 |
| A | 0 | 0 | A | 200 | 200 |
| N | 100 | 0 | N | 100 | 0 |
| A | 0 | 400 | A | 0 | 0 |
| S | 0 | 0 | S | 0 | 0 |
|  | 800 | 1500 |  | 800 | 850 |

- What size CARP drop zone is required for 2 C130Js if the lead aircraft is dropping 1 heavy equipment platforms and the second aircraft is dropping 72 jumpers and 2 door bundles mass exit flying NIT using SKE using GPS?

|  | W | L |  | W | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 600 | 1000 | 1 | 600 | 600 |
| N | 0 | 0 | N | 0 | 0 |
| A | 0 | 0 | A | 30 | 30 |
| N | 0 | 0 | N | 0 | 0 |
| A | 0 | 0 | A | 0 | 2775 |
| S | 400 | 0 | S | 400 | 0 |
|  | 1000 | 1000 |  | 1030 | 3405 |

- What size CARP drop zone is required for 2 C130Js if the lead aircraft is dropping 6 CDS bundles on a single roller system and the second aircraft is dropping 57 jumpers and 3 door bundles mass exit at night flying NIT using SKE?

|  | W | L |  | W | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 400 | 700 | 1 | 600 | 600 |
| N | 100 | 100 | N | 100 | 100 |
| A | 160 | 160 | A | 0 | 0 |
| N | 0 | 0 | N | 0 | 0 |
| A | 0 | 0 | A | 0 | 2250 |
| S | 400 | 0 | S | 400 | 0 |
|  | 1060 | 960 |  | 1100 | 2950 |

- What size CARP drop zone is required for 2 C130Js if the lead aircraft is dropping 3 heavy equipment platforms and the second aircraft is dropping 4 CDS bundles on a single roller system flying staggered trail?

|  | W | L |  | W | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 600 | 1000 | 1 | 400 | 550 |
| N | 0 | 0 | N | 0 | 0 |
| A | 0 | 0 | A | 200 | 200 |
| N | 100 | 0 | N | 100 | 0 |
| A | 0 | 800 | A | 0 | 0 |
| S | 0 | 0 | S | 0 | 0 |
|  | 700 | 1800 |  | 700 | 750 |

- What size CARP drop zone is required for 2 C130Hs if the lead aircraft is dropping 3 heavy equipment platforms and the second aircraft is dropping 49 jumpers and 2 door bundles mass exit at night using SKE from 1300 feet AGL?

|  | W | L |  | W | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 600 | 1000 | 1 | 600 | 600 |
| N | 100 | 100 | N | 100 | 100 |
| A | 60 | 60 | A | 90 | 90 |
| N | 0 | 0 | N | 0 | 0 |
| A | 0 | 800 | A | 0 | 1875 |
| S | 400 | 0 | S | 400 | 0 |
|  | 1160 | 1960 |  | 1190 | 2665 |

- What size CARP drop zone is required for 2 C130Js if the lead aircraft is dropping  10 CDS bundles on a double roller system and the second aircraft is dropping 14 jumpers and 2 door bundles at night from 1650 feet AGL flying NIT using SKE and GPS?

|  | W | L |  | W | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 400 | 700 | 1 | 600 | 600 |
| N | 100 | 100 | N | 100 | 100 |
| A | 400 | 400 | A | 180 | 180 |
| N | 0 | 0 | N | 0 | 0 |
| A | 0 | 0 | A | 0 | 1125 |
| S | 400 | 0 | S | 400 | 0 |
|  | 1300 | 1200 |  | 1280 | 2005 |

- What size CARP drop zone is required for 2 C130Hs if the lead aircraft is dropping 2 heavy equipment platforms and the second aircraft is dropping 3 CDS bundles on a single roller system at night from 1300 feet AGL flying staggered trail using SKE?

|  | W | L |  | W | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 600 | 1000 | 1 | 400 | 500 |
| N | 100 | 100 | N | 100 | 100 |
| A | 60 | 60 | A | 280 | 280 |
| N | 0 | 0 | N | 0 | 0 |
| A | 0 | 400 | A | 0 | 0 |
| S | 400 | 0 | S | 400 | 0 |
|  | 1160 | 1560 |  | 1180 | 880 |

- What size CARP drop zone is required for 2 C130Js if the lead aircraft is dropping 3 heavy equipment platforms and the second aircraft is dropping 54 jumpers and 2 door bundles mass exit at night from 1800 feet AGL flying NIT using SKE?

|  | W | L |  | W | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 600 | 1000 | 1 | 600 | 600 |
| N | 100 | 100 | N | 100 | 100 |
| A | 210 | 210 | A | 240 | 240 |
| N | 0 | 0 | N | 0 | 0 |
| A | 0 | 800 | A | 0 | 2100 |
| S | 400 | 0 | S | 400 | 0 |
|  | 1310 | 2110 |  | 1340 | 3040 |

- What size CARP drop zone is required for 3 C130Js if the first aircraft is dropping 5 heavy equipment, the second aircraft is dropping 6 CDS bundles on a single roller system and the third aircraft is dropping 64 jumpers and 2 door bundles mass exit at night using GPS?

|  | W | L |  |  | W | L |  |  | W | L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 600 | 1000 |  | 1 | 400 | 700 |  | 1 | 600 | 600 |
| N | 100 | 100 |  | N | 100 | 100 |  | N | 100 | 100 |
| A | 0 | 0 |  | A | 200 | 200 |  | A | 30 | 30 |
| N | 0 | 0 |  | N | 0 | 0 |  | N | 0 | 0 |
| A | 0 | 1600 |  | A | 0 | 0 |  | A | 0 | 2475 |
| S | 0 | 0 |  | S | 0 | 0 |  | S | 0 | 0 |
|  | 700 | 2700 |  |  | 700 | 1000 |  |  | 730 | 3205 |

- What size CARP drop zone is required for 3 C130Js if the first aircraft is dropping 3 heavy equipment, the second aircraft is dropping 9 CDS bundles on a double roller system and the third aircraft is dropping 54 jumpers and 2 door bundles mass exit at night flying NIT?

|  | W | L |  |  | W | L |  |  | W | L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 600 | 1000 |  | 1 | 400 | 700 |  | 1 | 600 | 600 |
| N | 100 | 100 |  | N | 100 | 100 |  | N | 100 | 100 |
| A | 0 | 0 |  | A | 200 | 200 |  | A | 30 | 30 |
| N | 100 | 0 |  | N | 100 | 0 |  | N | 100 | 0 |
| A | 0 | 800 |  | A | 0 | 0 |  | A | 0 | 2100 |
| S | 0 | 0 |  | S | 0 | 0 |  | S | 0 | 0 |
|  | 800 | 1900 |  |  | 800 | 1000 |  |  | 830 | 2830 |

- What size CARP drop zone is required for 3 C130Hs if the first aircraft is dropping 2 heavy equipment, the second aircraft is dropping 4 CDS bundles on a single roller system and the third aircraft is dropping 46 jumpers and 3 door bundles mass exit at night flying NIT?

|  | W | L |  |  | W | L |  |  | W | L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 600 | 1000 |  | 1 | 400 | 550 |  | 1 | 600 | 600 |
| N | 100 | 100 |  | N | 100 | 100 |  | N | 100 | 100 |
| A | 0 | 0 |  | A | 200 | 200 |  | A | 30 | 30 |
| N | 100 | 0 |  | N | 100 | 0 |  | N | 100 | 0 |
| A | 0 | 400 |  | A | 0 | 0 |  | A | 0 | 1800 |
| S | 0 | 0 |  | S | 0 | 0 |  | S | 0 | 0 |
|  | 800 | 1500 |  |  | 800 | 850 |  |  | 830 | 2530 |

- What size CARP drop zone is required for 3 C130Js if the first aircraft is dropping 4 heavy equipment, the second aircraft is dropping 10 CDS bundles on a double roller system and the third aircraft is dropping 76 jumpers and 2 door bundles mass exit using SKE and GPS?

|  | W | L |  |  | W | L |  |  | W | L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 600 | 1000 |  | 1 | 400 | 700 |  | 1 | 600 | 600 |
| N | 0 | 0 |  | N | 0 | 0 |  | N | 0 | 0 |
| A | 0 | 0 |  | A | 200 | 200 |  | A | 30 | 30 |
| N | 0 | 0 |  | N | 0 | 0 |  | N | 0 | 0 |
| A | 0 | 1200 |  | A | 0 | 0 |  | A | 0 | 2925 |
| S | 400 | 0 |  | S | 400 | 0 |  | S | 400 | 0 |
|  | 1000 | 2200 |  |  | 1000 | 900 |  |  | 1030 | 3555 |

- What size CARP drop zone is required for 3 C130Hs if the first aircraft is dropping 5 heavy equipment, the second aircraft is dropping 2 CDS bundles on a single roller system and the third aircraft is dropping 78 jumpers and 2 door bundles mass exit using SKE from 800 feet AGL?

|  | W | L |  |  | W | L |  |  | W | L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 600 | 1000 |  | 1 | 400 | 450 |  | 1 | 600 | 600 |
| N | 0 | 0 |  | N | 0 | 0 |  | N | 0 | 0 |
| A | 0 | 0 |  | A | 80 | 80 |  | A | 0 | 0 |
| N | 0 | 0 |  | N | 0 | 0 |  | N | 0 | 0 |
| A | 0 | 1600 |  | A | 0 | 0 |  | A | 0 | 3000 |
| S | 400 | 0 |  | S | 400 | 0 |  | S | 400 | 0 |
|  | 1000 | 2600 |  |  | 880 | 530 |  |  | 1000 | 3600 |

- What size CARP drop zone is required for 3 C130Js if the first aircraft is dropping 1 heavy equipment, the second aircraft is dropping 6 CDS bundles on a double roller system and the third aircraft is dropping 57 jumpers and 1 door bundles mass exit at night flying NIT using GPS from 1300 feet AGL?

|  | W | L |  |  | W | L |  |  | W | L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 600 | 1000 |  | 1 | 400 | 500 |  | 1 | 600 | 600 |
| N | 100 | 100 |  | N | 100 | 100 |  | N | 100 | 100 |
| A | 60 | 60 |  | A | 280 | 280 |  | A | 90 | 90 |
| N | 100 | 0 |  | N | 100 | 0 |  | N | 100 | 0 |
| A | 0 | 0 |  | A | 0 | 0 |  | A | 0 | 2175 |
| S | 0 | 0 |  | S | 0 | 0 |  | S | 0 | 0 |
|  | 860 | 1160 |  |  | 880 | 880 |  |  | 890 | 2965 |

- What size CARP drop zone is required for 3 C130Js if the first aircraft is dropping 5 heavy equipment, the second aircraft is dropping 2 CDS bundles on a single roller system and the third aircraft is dropping 28 jumpers and 2door bundles at night from 1750 feet AGL flying NIT using SKE?

|  | W | L |  |  | W | L |  |  | W | L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 600 | 1000 |  | 1 | 400 | 450 |  | 1 | 600 | 600 |
| N | 100 | 100 |  | N | 100 | 100 |  | N | 100 | 100 |
| A | 180 | 180 |  | A | 440 | 440 |  | A | 210 | 210 |
| N | 0 | 0 |  | N | 0 | 0 |  | N | 0 | 0 |
| A | 0 | 1600 |  | A | 0 | 0 |  | A | 0 | 2175 |
| S | 400 | 0 |  | S | 400 | 0 |  | S | 400 | 0 |
|  | 1280 | 2880 |  |  | 1340 | 990 |  |  | 1310 | 3085 |

- What size CARP drop zone is required for 3 C130Js if the first aircraft is dropping 3 heavy equipment, the second aircraft is dropping 8 CDS bundles on a double roller system and the third aircraft is dropping 32 jumpers and 3 door bundles mass exit at night from 1000 feet AGL flying NIT using SKE?

|  | W | L |  |  | W | L |  |  | W | L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 600 | 1000 |  | 1 | 400 | 550 |  | 1 | 600 | 600 |
| N | 100 | 100 |  | N | 100 | 100 |  | N | 100 | 100 |
| A | 0 | 0 |  | A | 160 | 160 |  | A | 0 | 0 |
| N | 0 | 0 |  | N | 0 | 0 |  | N | 0 | 0 |
| A | 0 | 800 |  | A | 0 | 0 |  | A | 0 | 1275 |
| S | 400 | 0 |  | S | 400 | 0 |  | S | 400 | 0 |
|  | 1100 | 1900 |  |  | 1060 | 810 |  |  | 1100 | 1975 |

- What size CARP drop zone is required for 3 C130Js if the first aircraft is dropping 2 heavy equipment, the second aircraft is dropping 4 CDS bundles on a single roller system and the third aircraft is dropping 43 jumpers and 2 door bundles mass exit at night from 1250 feet AGL flying NIT using SKE and GPS?

|  | W | L |  |  | W | L |  |  | W | L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 600 | 1000 |  | 1 | 400 | 550 |  | 1 | 600 | 600 |
| N | 100 | 100 |  | N | 100 | 100 |  | N | 100 | 100 |
| A | 30 | 30 |  | A | 240 | 240 |  | A | 60 | 60 |
| N | 0 | 0 |  | N | 0 | 0 |  | N | 0 | 0 |
| A | 0 | 400 |  | A | 0 | 0 |  | A | 0 | 1650 |
| S | 400 | 0 |  | S | 400 | 0 |  | S | 400 | 0 |
|  | 1130 | 1530 |  |  | 1140 | 890 |  |  | 1160 | 2410 |

- What size CARP drop zone is required for 3 C130Hs if the first aircraft is dropping 3 heavy equipment, the second aircraft is dropping 3 CDS bundles on a single roller system and the third aircraft is dropping 62 jumpers and 1 door bundle mass exit at night from1600 feet AGL flying NIT using SKE?

|  | W | L |  |  | W | L |  |  | W | L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 600 | 1000 |  | 1 | 400 | 500 |  | 1 | 600 | 600 |
| N | 100 | 100 |  | N | 100 | 100 |  | N | 100 | 100 |
| A | 150 | 150 |  | A | 400 | 400 |  | A | 180 | 180 |
| N | 0 | 0 |  | N | 0 | 0 |  | N | 0 | 0 |
| A | 0 | 800 |  | A | 0 | 0 |  | A | 0 | 2325 |
| S | 400 | 0 |  | S | 400 | 0 |  | S | 400 | 0 |
|  | 1250 | 2050 |  |  | 1300 | 1000 |  |  | 1280 | 3205 |

- What size CARP drop zone is required for 1 C17 dropping 100 jumpers and 2 door bundles each mass exit at night?

|  | W | L |
| --- | --- | --- |
| 1 | 600 | 600 |
| N | 100 | 100 |
| A | 0 | 0 |
| N | 0 | 0 |
| A | 0 | 3825 |
| S | 0 | 0 |
|  | 700 | 4525 |

- What size CARP drop zone is required for 2 C17s dropping 5 CDS bundles on a single roller system each at night using GPS?

|  | W | L |
| --- | --- | --- |
| 1 | 450 | 765 |
| N | 0 | 100 |
| A | 0 | 0 |
| N | 0 | 0 |
| A | 0 | 0 |
| S | 0 | 0 |
|  | 450 | 865 |

- What size CARP drop zone is required for 2 C17s dropping 6 heavy equipment platforms each at night flying staggered trail?

|  | W | L |
| --- | --- | --- |
| 1 | 600 | 1000 |
| N | 100 | 100 |
| A | 0 | 0 |
| N | 100 | 0 |
| A | 0 | 2500 |
| S | 0 | 0 |
|  | 800 | 3600 |

- What size CARP drop zone is required for 3 C17s dropping 4 CDS bundles on a single roller system each at night from 1600 feet AGL flying NIT using SKE and GPS?

|  | W | L |
| --- | --- | --- |
| 1 | 450 | 765 |
| N | 0 | 100 |
| A | 400 | 400 |
| N | 0 | 0 |
| A | 0 | 0 |
| S | 400 | 0 |
|  | 1250 | 1265 |

- What size CARP drop zone is required for 2 C17s dropping 5 heavy equipment platforms on a dual row delivery system each at night from 1800 feet AGL flying NIT using SKE?

|  | W | L |
| --- | --- | --- |
| 1 | 600 | 1000 |
| N | 100 | 100 |
| A | 180 | 180 |
| N | 0 | 0 |
| A | 0 | 1600 |
| S | 400 | 0 |
|  | 1280 | 2880 |

- What size CARP drop zone is required for 3 C17s dropping 84 jumpers and 1 door bundles each mass exit at night from 1250 feet AGL on surveyed PI using GPS?

|  | W | L |
| --- | --- | --- |
| 1 | 600 | 600 |
| N | 0 | 100 |
| A | 60 | 60 |
| N | 1200 | 0 |
| A | 0 | 3150 |
| S | 0 | 0 |
|  | 1860 | 3910 |

- What size CARP drop zone is required for 3 C17s dropping 18 CDS bundles on a single roller system each at night from 1000 feet AGL flying NIT using SKE?

|  | W | L |
| --- | --- | --- |
| 1 | 450 | 1065 |
| N | 100 | 100 |
| A | 160 | 160 |
| N | 0 | 0 |
| A | 0 | 0 |
| S | 400 | 0 |
|  | 1110 | 1325 |

- What size CARP drop zone is required for 3 C17s dropping 3 heavy equipment platforms each at night from 1300 feet AGL flying NIT using GPS?

|  | W | L |
| --- | --- | --- |
| 1 | 600 | 1000 |
| N | 0 | 100 |
| A | 60 | 60 |
| N | 100 | 0 |
| A | 0 | 1000 |
| S | 0 | 0 |
|  | 760 | 2160 |

- What size CARP drop zone is required for 2 C17s dropping 28 jumpers and 3 door bundles each at night from 900 feet AGL on off-set PIs using GPS?

|  | W | L |
| --- | --- | --- |
| 1 | 600 | 600 |
| N | 0 | 100 |
| A | 0 | 0 |
| N | 450 | 0 |
| A | 0 | 2250 |
| S | 0 | 0 |
|  | 1050 | 2950 |

- What size CARP drop zone is required for 2 C17s dropping 33 jumpers and 3 door bundles each mass exit at night from 1500 feet AGL on the center PI?

|  | W | L |
| --- | --- | --- |
| 1 | 600 | 600 |
| N | 100 | 100 |
| A | 150 | 150 |
| N | 640 | 0 |
| A | 0 | 1350 |
| S | 0 | 0 |
|  | 1490 | 2200 |

- What size CARP drop zone is required for 2 C17s separated by 20 minutes if the lead aircraft is dropping 7 heavy equipment platforms from 1200 feet AGL and the second aircraft is dropping 100 jumpers and 2 door bundles mass exit from planning altitude for a tactical training jump at night with both aircraft using GPS?

|  | W | L |  | W | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 600 | 1000 | 1 | 600 | 600 |
| N | 0 | 100 | N | 0 | 100 |
| A | 30 | 30 | A | 0 | 0 |
| N | 0 | 0 | N | 0 | 0 |
| A | 0 | 3000 | A | 0 | 3825 |
| S | 0 | 0 | S | 0 | 0 |
|  | 630 | 4130 |  | 600 | 4525 |

- What size CARP drop zone is required for 2 C17s separated by 15 minutes if the lead aircraft is dropping 2 CDS bundles on a single roller system from 900 feet AGL and the second aircraft is dropping 52 jumpers and 3 door bundles mass exit at night?

|  | W | L |  | W | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 450 | 615 | 1 | 600 | 600 |
| N | 100 | 100 | N | 100 | 100 |
| A | 120 | 120 | A | 0 | 0 |
| N | 0 | 0 | N | 0 | 0 |
| A | 0 | 0 | A | 0 | 2025 |
| S | 0 | 0 | S | 0 | 0 |
|  | 670 | 835 |  | 700 | 2725 |

- What size CARP drop zone is required for 2 C17s separated by 15 minutes if the lead aircraft is dropping 13 CDS bundles on a double roller system from 1200 feet AGL and the second aircraft is dropping 33 jumpers and 2 door bundles at night with the 2nd aircraft using GPS?

|  | W | L |  | W | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 450 | 765 | 1 | 600 | 600 |
| N | 100 | 100 | N | 0 | 100 |
| A | 240 | 240 | A | 0 | 0 |
| N | 0 | 0 | N | 0 | 0 |
| A | 0 | 0 | A | 0 | 2550 |
| S | 0 | 0 | S | 0 | 0 |
|  | 790 | 1105 |  | 600 | 3250 |

- What size CARP drop zone is required for 2 C17s if the lead aircraft is dropping 4 heavy equipment platforms on a dual row delivery system and the second aircraft is dropping 17 CDS bundles on a single roller system using SKE?

|  | W | L |  | W | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 600 | 1000 | 1 | 450 | 1065 |
| N | 0 | 0 | N | 0 | 0 |
| A | 0 | 0 | A | 240 | 240 |
| N | 0 | 0 | N | 0 | 0 |
| A | 0 | 1200 | A | 0 | 0 |
| S | 400 | 0 | S | 400 | 0 |
|  | 1000 | 2200 |  | 1090 | 1305 |

- What size CARP drop zone is required for 2 C17s separated by 15 minutes if the lead aircraft is dropping 2 heavy equipment platforms from 1500 feet AGL and the second aircraft is dropping 65 jumpers and 3 door bundles mass exit from 1200 feet AGL at night with both aircraft using GPS?

|  | W | L |  | W | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 600 | 1000 | 1 | 600 | 600 |
| N | 0 | 100 | N | 0 | 100 |
| A | 120 | 120 | A | 60 | 60 |
| N | 0 | 0 | N | 0 | 0 |
| A | 0 | 500 | A | 0 | 2550 |
| S | 0 | 0 | S | 0 | 0 |
|  | 720 | 1720 |  | 660 | 3310 |

- What size CARP drop zone is required for 2 C17s separated by 20 minutes if the lead aircraft is dropping 6 CDS bundles on a single roller system and the second aircraft is dropping 26 jumpers and 2 door bundles mass exit?

|  | W | L |  | W | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 450 | 765 | 1 | 600 | 600 |
| N | 0 | 0 | N | 0 | 0 |
| A | 0 | 0 | A | 0 | 0 |
| N | 0 | 0 | N | 0 | 0 |
| A | 0 | 0 | A | 0 | 1050 |
| S | 0 | 0 | S | 0 | 0 |
|  | 450 | 765 |  | 600 | 1650 |

- What size CARP drop zone is required for 2 C17s if the lead aircraft is dropping 2 heavy equipment platforms on a dual row delivery system and the second aircraft is dropping 2 CDS bundles on a single roller system at night flying NIT using SKE and GPS?

|  | W | L |  | W | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 600 | 1000 | 1 | 450 | 615 |
| N | 0 | 100 | N | 0 | 100 |
| A | 0 | 0 | A | 240 | 240 |
| N | 0 | 0 | N | 0 | 0 |
| A | 0 | 400 | A | 0 | 0 |
| S | 400 | 0 | S | 400 | 0 |
|  | 1000 | 1500 |  | 1090 | 955 |

- What size CARP drop zone is required for 2 C17s if the lead aircraft is dropping 5 heavy equipment platforms and the trail aircraft is dropping 38 CDS on a double roller system from 1500 feet AGL at night flying staggered trail?

|  | W | L |  | W | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 600 | 1000 | 1 | 450 | 1065 |
| N | 100 | 100 | N | 100 | 100 |
| A | 120 | 120 | A | 360 | 360 |
| N | 100 | 0 | N | 100 | 0 |
| A | 0 | 2000 | A | 0 | 0 |
| S | 0 | 0 | S | 0 | 0 |
|  | 920 | 3220 |  | 1010 | 1525 |

- What size CARP drop zone is required for 2 C17s separated by 15 minutes if the first aircraft is dropping 17 CDS bundles on a double roller system from 1000 feet AGL and the second aircraft is dropping 19 jumpers and 2 door bundles mass exit at night from 1250 feet AGL using with the second aircraft using GPS?

|  | W | L |  | W | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 450 | 915 | 1 | 600 | 600 |
| N | 100 | 100 | N | 0 | 100 |
| A | 160 | 160 | A | 60 | 60 |
| N | 0 | 0 | N | 0 | 0 |
| A | 0 | 0 | A | 0 | 750 |
| S | 0 | 0 | S | 0 | 0 |
|  | 710 | 1175 |  | 660 | 1510 |

- What size CARP drop zone is required for 2 C17s separated by 15 minutes if the lead aircraft is dropping 4 heavy equipment platforms on a dual row delivery system from 1500 feet AGL and the second aircraft is dropping 84 jumpers and 2 door bundles mass exit from 1800 feet AGL at night with both aircraft using GPS?

|  | W | L |  | W | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 600 | 1000 | 1 | 600 | 600 |
| N | 0 | 100 | N | 0 | 100 |
| A | 90 | 90 | A | 240 | 240 |
| N | 0 | 0 | N | 0 | 0 |
| A | 0 | 1200 | A | 0 | 3225 |
| S | 0 | 0 | S | 0 | 0 |
|  | 690 | 2390 |  | 840 | 4165 |

- What size CARP drop zone is required for a C130J dropping 4 rows of HVCDS at night using GPS?

|  | W | L |
| --- | --- | --- |
| 1 | 580 | 660 |
| N | 0 | 100 |
| A | 0 | 0 |
| N | 0 | 0 |
| A | 0 | 150 |
| S | 0 | 0 |
|  | 580 | 910 |

- What size CARP drop zone is required for a C17 dropping 15 HAARS containers from 3200 feet AGL at night?

|  | W | L |
| --- | --- | --- |
| 1 | 500 | 1900 |
| N | 100 | 100 |
| A | 0 | 0 |
| N | 0 | 0 |
| A | 0 | 0 |
| S | 0 | 0 |
|  | 600 | 2000 |

- What size CARP drop zone is required for a C130J dropping 1 row of HV-LCADS from 5000 feet AGL?

|  | W | L |
| --- | --- | --- |
| 1 | 580 | 660 |
| N | 0 | 0 |
| A | 100 | 400 |
| N | 0 | 0 |
| A | 0 | 0 |
| S | 0 | 0 |
|  | 680 | 1060 |

- What size CARP drop zone is required for 2 C130Js if the lead aircraft is dropping 3 rows of HVCDS and the trail aircraft is dropping 4 HAARS containers from 4500 feet AGL flying staggered trail at night using GPS?

|  | W | L |  | W | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 580 | 660 | 1 | 500 | 1200 |
| N | 100 | 100 | N | 100 | 100 |
| A | 50 | 200 | A | 50 | 100 |
| N | 100 | 0 | N | 100 | 0 |
| A | 0 | 100 | A | 0 | 0 |
| S | 0 | 0 | S | 0 | 0 |
|  | 830 | 1060 |  | 750 | 1400 |

- What size CARP drop zone is required for 3 C17s if the lead aircraft is dropping 12 rows of HVCDS and the remaining two aircraft are dropping 20 HAARS containers each from 5000 feet AGL at night flying NIT using SKE and GPS?

|  | W | L |  | W | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 580 | 660 | 1 | 500 | 1900 |
| N | 0 | 100 | N | 0 | 100 |
| A | 100 | 400 | A | 100 | 200 |
| N | 0 | 0 | N | 0 | 0 |
| A | 0 | 550 | A | 0 | 0 |
| S | 400 | 0 | S | 400 | 0 |
|  | 1080 | 1710 |  | 1000 | 2200 |

- What size CARP drop zone is required for a C17 delivering a JPADS platform?

328 Yard Radius

- What size CARP drop zone is required for a C130J dropping 6 rows of HV-LCADS from 2500 feet AGL at night using GPS?

|  | W | L |
| --- | --- | --- |
| 1 | 580 | 660 |
| N | 0 | 100 |
| A | 0 | 0 |
| N | 0 | 0 |
| A | 0 | 250 |
| S | 0 | 0 |
|  | 580 | 1010 |

- What size CARP drop zone is required for a 2 C130Js dropping 7 HAARS containers each from 6000 feet AGL at night using SKE?

|  | W | L |
| --- | --- | --- |
| 1 | 500 | 1200 |
| N | 100 | 100 |
| A | 150 | 300 |
| N | 0 | 0 |
| A | 0 | 0 |
| S | 400 | 0 |
|  | 1150 | 1600 |

- What size CARP drop zone is required for 3 C17s dropping 20 rows of HV-CDS each from 5000 feet AGL at night flying NIT using SKE?

|  | W | L |
| --- | --- | --- |
| 1 | 580 | 660 |
| N | 100 | 100 |
| A | 100 | 400 |
| N | 0 | 0 |
| A | 0 | 950 |
| S | 400 | 0 |
|  | 1180 | 2110 |

- What size CARP drop zone is required for a C130H dropping a JPADS platform from 17,500 feet AGL?

765 yard radius

Formulas

D=KxAxV

- What is the drift that the DZSTL will pace off for a UH-60 dropping door bundles during the day with a MEW of 12 KIAS?

2.  What is the drift that the DZSTL will pace off for a UH-60 dropping door bundles at night with a MEW of 4 KIAS?

3.  What is the drift that the DZSTL will pace off for a CH-47 dropping jumpers at night with a MEW of 11 KIAS?

4.  What is the drift that the DZSTL will pace off for a CH-47 dropping jumpers at night from the minimum drop altitude with a MEW of 12 KIAS?

5.  What is the drift that the DZSTL will pace off for a C130 dropping door bundles with the T-10 cargo parachute at night from the minimum drop altitude with a MEW of 8 KIAS?

6.  What is the drift that the DZSTL will pace off for a C130 dropping door bundles with the G14 parachute at night from the minimum drop altitude with a MEW of 2 KIAS?

7.  What is the drift that the DZSTL will pace off for a C17 dropping door bundles with the T-10 cargo parachute at night from the minimum drop altitude with a MEW of 18 KIAS?

8.  What is the drift that the DZSTL will pace off for a C17 dropping door bundles with the G14 parachute at night from the minimum drop altitude with a MEW of 12 KIAS?

9.  What is the drift that the DZSTL will pace off for a C130 dropping door bundles using the T-10 cargo parachute at night with a MEW of 14 KIAS?

10.  What is the drift that the DZSTL will pace off for a C130 dropping CDS with the G14 parachute at night from the minimum drop altitude with a MEW of 23 KIAS?

11.  What is the drift that the DZSTL will pace off for a C17 dropping CDS with the G14 parachute at night from the minimum drop altitude with a MEW of 13 KIAS?

12.  What is the drift that the DZSTL will pace off for a C130 dropping CDS using the G14 parachute at night with a MEW of 6 KIAS?

13.  What is the drift that the DZSTL will pace off for a C130 dropping heavy equipment with the G12E parachute at night from the minimum drop altitude with a MEW of 3 KIAS?

14.  What is the drift that the DZSTL will pace off for a C17 dropping heavy equipment with the 5000 lbs parachute release system at night from the minimum drop altitude with a MEW of 14 KIAS?

15.  What is the drift that the DZSTL will pace off for a C130 dropping heavy equipment using the G12E parachute at night with a MEW of 11 KIAS?

16.  What is the drift that the DZSTL will pace off for a C130 dropping jumpers conducting combat training at night with a MEW of 15 KIAS?

17.  What is the drift that the DZSTL will pace off for a C130 dropping jumpers at night with a MEW of 9 KIAS?

18.  What is the drift that the DZSTL will pace off for a C130 dropping SATB-Ps from 500 feet AGL with a MEW of 4 KIAS?

19.  What is the drift that the DZSTL will pace off for a C130 dropping basic airborne students at night with a MEW of 23 KIAS?

20.  What is the drift that the DZSTL will pace off for a C17 dropping jumpers and heavy equipment at night with a MEW of 8 KIAS?

21.  What is the drift that the DZSTL will pace off for a C17 dropping jumpers and CDS at night with a MEW of 22 KIAS?

22.  What is the drift that the DZSTL will pace off for a C130 dropping CDS and heavy equipment at night with a MEW of 19 KIAS?

23.  What is the drift that the DZSTL will pace off for a C17 dropping door bundles and CDS at night with a MEW of 13 KIAS?

24.  What is the drift that the DZSTL will pace off for a C130 dropping door bundles and heavy equipment using the 5000 lbs release system at night with a MEW of 17 KIAS?

25.  What is the drift that the DZSTL will pace off for a C130 dropping jumpers and door bundles at night with a  MEW of 11 KIAS?

26.  What is the drift that the DZSTL will pace off for a UH-60 dropping jumpers and door bundles with a MEW of 9 KIAS?

27.  What is the drift that the DZSTL will pace off for a C130 dropping CDS from 3000 feet AGL with a MEW of 7 KIAS?

28.  What is the drift that the DZSTL will pace off for a C17 dropping jumpers from 2500 feet AGL with a MEW of 11 KIAS?

29.  What is the drift that the DZSTL will pace off for a CH-47 dropping bundles from 1800 feet AGL with a MEW of 23 KIAS?

30.  What is the drift that the DZSTL will pace off for a C130 dropping SATB-Ps from 1100 feet AGL with a MEW of 3 KIAS?

31.  What is the drift that the DZSTL will pace off for a C17 dropping CDS and heavy equipment with a MEW of 29 KIAS?

32.  What is the drift that the DZSTL will pace off for a C130 dropping jumpers and heavy equipment with a MEW of 12 KIAS?

33.  What is the drift that the DZSTL will pace off for a C130 dropping CDS with a MEW of 16 KIAS?

34.  What is the drift that the DZSTL will pace off for a CH-47 dropping jumpers from 2500 feet AGL with a MEW of 5 KIAS?

35.  What is the drift that the DZSTL will pace off for a C130 dropping jumpers and door bundles from 1250 feet AGL with a MEW of 7 KIAS?

36.  What is the drift that the DZSTL will pace off for a UH-60 dropping door bundles from 2100 feet AGL with a MEW of 6 KIAS?

37.  What is the drift that the DZSTL will pace off for a C17 dropping heavy equipment from 800 feet AGL with a MEW of 13 KIAS?

38.  What is the drift that the DZSTL will pace off for a C130 dropping jumpers from 1450 feet AGL with a MEW of 22 KIAS?

39.  What is the drift that the DZSTL will pace off for a C130 dropping CDS from 1300 feet AGL with a MEW of 14 KIAS?

40.  What is the drift that the DZSTL will pace off for a C130 dropping CDS from 1800 feet AGL with a MEW of 9 KIAS?

41.  What is the drift that the DZSTL will pace off for a C17 dropping heavy equipment from 600 feet AGL with a MEW of 4 KIAS?

42.  What is the drift that the DZSTL will pace off for a C17 dropping jumpers and door bundles from 1300 feet AGL with a MEW of 13 KIAS?

43.  What is the drift that the DZSTL will pace off for a C17 dropping jumpers and door bundles from 2300 feet AGL with a MEW of 17 KIAS?

44.  What is the drift that the DZSTL will pace off for a C130 dropping SATB-Ps from 1000 feet AGL a MEW of 6 KIAS?

45.  What is the drift that the DZSTL will pace off for a C17 dropping SATB-Ps and door bundles from 1250 feet AGL with a MEW of 4 KIAS?

46.  What is the drift that the DZSTL will pace off for a UH-60 dropping and door bundles from 3000 feet AGL with a MEW of 15 KIAS?

47.  What is the drift that the DZSTL will pace off for a C130 dropping CDS and jumpers from 950 feet AGL with a MEW of 10 KIAS?

48.  What is the drift that the DZSTL will pace off for a C17 dropping CDS and jumpers from 1300 feet AGL with a MEW of 11 KIAS?

49.  What is the drift that the DZSTL will pace off for a C130 dropping door bundles and jumpers from 1850 feet AGL with a MEW of 12 KIAS?

50.  What is the drift that the DZSTL will pace off for a C17 dropping door bundles and jumpers from 3000 feet AGL with a MEW of 17 KIAS?

Formulas

D=RxT

1.  A C130 is dropping 25 jumpers on a GMRS drop zone. What length drop zone is required to get all the jumpers out in one pass?

2.  A C17 is dropping 28 jumpers on a GMRS drop zone. What length drop zone is required to get all the jumpers out in one pass?

3.  A UH-1 is dropping 6 jumpers on a VIRS drop zone. What length drop zone is required to get all the jumpers out in one pass?

4.  A UH-60 is dropping 8 jumpers on a VIRS drop zone. What length drop zone is required to get all the jumpers out in one pass?

5.  A CH-47 is dropping 27 jumpers on a VIRS drop zone. What length drop zone is required to get all the jumpers out in one pass?

6.  A C130 is dropping 10 door bundles on a GMRS drop zone. What length drop zone is required to get all the door bundles out in one pass?

7.  A C17 is dropping 14 door bundles on a GMRS drop zone. What length drop zone is required to get all the door bundles out in one pass?

8.  A UH-1 is dropping 4 door bundles on a VIRS drop zone. What length drop zone is required to get all the door bundles out in one pass?

9.  A UH-60 is dropping 5 door bundles on a VIRS drop zone. What length drop zone is required to get all the door bundles out in one pass?

10.  A CH-47 is dropping 13 door bundles on a VIRS drop zone. What length drop zone is required to get all the door bundles out in one pass?

11.  A C130 is dropping 30 jumpers and 3 door bundles on a GMRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass?

12.  A C17 is dropping 41 jumpers and 5 door bundles on a GMRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass?

13.  A UH-1 is dropping 4 jumpers and 2 door bundles on a VIRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass?

14.  A UH-60 is dropping 3 jumpers and 3 door bundles on a VIRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass?

15.  A CH-47 is dropping 25 jumpers and 2 door bundles on a VIRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass?

16.  A C130 is dropping 64 jumpers and 2 door bundles mass exit on a GMRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass?

17.  A C17 is dropping 97 jumpers and 3 door bundles mass exit on a GMRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass?

18.  A UH-60 is dropping 5 jumpers and 1 door bundles on a VIRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass?

19.  A UH-60 is dropping 2 jumpers and 2 door bundles on a VIRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass?

20.  A CH-47 is dropping 21 jumpers and 3 door bundles on a VIRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass?

21.  A C130 flying 135 KIAS is dropping 61 jumpers and 1 door bundle mass exit on a GMRS drop zone. What length drop zone is required to get all the jumpers and door bundle out in one pass?

22.  A C17 flying 138 KIAS is dropping 91 jumpers and 2 door bundles mass exit on a GMRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass?

23.  A UH-60 flying 50 KIAS is dropping 7 jumpers on a VIRS drop zone. What length drop zone is required to get all the jumpers out in one pass?

24.  A UH-60 flying 75 KIAS is dropping 6 jumpers on a VIRS drop zone. What length drop zone is required to get all the jumpers out in one pass?

25.  A CH-47 flying 80 KIAS is dropping 17 jumpers on a VIRS drop zone. What length drop zone is required to get all the jumpers out in one pass?

26.  A C130 is dropping 53 jumpers and 2 door bundles mass exit on a GMRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass if the point of impact was moved forward 160 meters to fit the markings on the drop zone?

27.  A C17 is dropping 83 jumpers and 3 door bundles mass exit on a GMRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass if the point of impact was moved forward 185 meters to fit the markings on the drop zone?

28.  A UH-60 flying 60 KIAS is dropping 4 jumpers on a VIRS drop zone. What length drop zone is required to get all the jumpers out in one pass if the point of impact was moved forward 95 meters to fit the markings on the drop zone?

29.  A UH-60 flying 65 KIAS is dropping 3 jumpers on a VIRS drop zone. What length drop zone is required to get all the jumpers out in one pass if the point of impact was moved forward 65 meters to fit the markings on the drop zone?

30.  A CH-47 flying 85 KIAS is dropping 26 jumpers on a VIRS drop zone. What length drop zone is required to get all the jumpers out in one pass if the point of impact was moved forward 100 meters to fit the markings on the drop zone?

31.  A C130 is dropping 73 jumpers and 1 door bundle mass exit on a GMRS drop zone. What length drop zone is required to get all the jumpers and door bundle out in one pass if the point of impact was moved forward 135 meters to fit the markings on the drop zone?

32.  A C17 is dropping 94 jumpers and 1 door bundle mass exit on a GMRS drop zone. What length drop zone is required to get all the jumpers and door bundle out in one pass if the point of impact was moved forward 112 meters to fit the markings on the drop zone?

33.  A C130 is dropping 22 jumpers and 2 door bundles mass exit on a GMRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass if the point of impact was moved forward 120 meters to fit the markings on the drop zone?

34.  A C17 is dropping 98 jumpers and 2 door bundles mass exit on a GMRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass if the point of impact was moved forward 179 meters to fit the markings on the drop zone?

35.  A C130 is dropping 78 jumpers and 2 door bundles mass exit on a GMRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass if the point of impact was moved forward 140 meters to fit the markings on the drop zone?

36.  A C17 flying 135 KIAS is dropping 89 jumpers and 3 door bundles mass exit on a GMRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass if the point of impact was moved forward 107 meters to fit the markings on the drop zone?

37.  A C130 flying 135 KIAS is dropping 67 jumpers and 2 door bundles mass exit on a GMRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass if the point of impact was moved forward 315 meters to fit the markings on the drop zone?

38.  A C17 flying 135 KIAS is dropping 45 jumpers and 8 door bundles mass exit on a GMRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass if the point of impact was moved forward 212 meters to fit the markings on the drop zone?

39.  A C130 flying 135 KIAS is dropping 72 jumpers and 1door bundles mass exit on a GMRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass if the point of impact was moved forward 155 meters to fit the markings on the drop zone?

40.  A C17 flying 135 KIAS is dropping 88 jumpers and 2 door bundles mass exit on a GMRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass if the point of impact was moved forward 205 meters to fit the markings on the drop zone?

41.  A C130 is dropping 71 jumpers and 5 door bundles mass exit on a GMRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass if the point of impact was moved forward 108 meters to fit the markings on the drop zone?

42.  A C17 is dropping 76 jumpers and 4 door bundles mass exit on a GMRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass if the point of impact was moved forward 105 meters to fit the markings on the drop zone?

43.  A C130 is dropping 75 jumpers and 2 door bundles mass exit on a GMRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass if the point of impact was moved forward 128 meters to fit the markings on the drop zone?

44.  A C17 is dropping 98 jumpers and 1 door bundle mass exit on a GMRS drop zone. What length drop zone is required to get all the jumpers and door bundle out in one pass if the point of impact was moved forward 68 meters to fit the markings on the drop zone?

45.  A C130 is dropping 61 jumpers and 3 door bundles mass exit on a GMRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass if the point of impact was moved forward 160 meters to fit the markings on the drop zone?

46.  A C17 flying 135 KIAS is dropping 89 jumpers and 2 door bundles mass exit on a GMRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass if the point of impact was moved forward 320 meters to fit the markings on the drop zone?

47.  A C130 flying 135 KIAS is dropping 69 jumpers and 2 door bundles mass exit on a GMRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass if the point of impact was moved forward 95 meters to fit the markings on the drop zone?

48.  A C17 flying 135 KIAS is dropping 96 jumpers and 3 door bundles mass exit on a GMRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass if the point of impact was moved forward 179 meters to fit the markings on the drop zone?

49.  A C130 flying 135 KIAS is dropping 45 jumpers and 2 door bundles mass exit on a GMRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass if the point of impact was moved forward 193 meters to fit the markings on the drop zone?

50.  A C17 flying 135 KIAS is dropping 92 jumpers and 5 door bundles mass exit on a GMRS drop zone. What length drop zone is required to get all the jumpers and door bundles out in one pass if the point of impact was moved forward 117 meters to fit the markings on the drop zone?

Formulas

T=D/R

1.  A C130 is dropping 25 jumpers on a GMRS drop zone that is 1100 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft?

2.  A C17 is dropping 28 jumpers on a GMRS drop zone that is 900 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft?

3.  A UH-60 is dropping 6 jumpers on a VIRS drop zone that is 300 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft?

4.  A UH-60 is dropping 8 jumpers on a VIRS drop zone that is 350 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft?

5.  A CH-47 is dropping 27 jumpers on a VIRS drop zone that is 800 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft?

6.  A C130 is dropping 10 door bundles on a GMRS drop zone that is 850 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft?

7.  A C17 is dropping 14 door bundles on a GMRS drop zone that is 900 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft?

8.  A UH-60 is dropping 4 door bundles on a VIRS drop zone that is 325 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft?

9.  A UH-60 is dropping 5 door bundles on a VIRS drop zone that is 350 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft?

10.  A CH-47 is dropping 13 door bundles on a VIRS drop zone that is 500 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft?

11.  A C130 is dropping 30 jumpers and 3 door bundles on a GMRS drop zone that is 950 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft?

12.  A C17 is dropping 41 jumpers and 5 door bundles on a GMRS drop zone that is 1200 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft?

13.  A UH-60 is dropping 4 jumpers and 2 door bundles on a VIRS drop zone that is 400 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft?

14.  A UH-60 is dropping 3 jumpers and 3 door bundles on a VIRS drop zone that is 450 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft?

15.  A CH-47 is dropping 25 jumpers and 2 door bundles on a VIRS drop zone that is 725 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft?

16.  A C130 is dropping 64 jumpers and 2 door bundles mass exit on a GMRS drop zone that is 1100 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft?

17.  A C17 is dropping 97 jumpers and 3 door bundles mass exit on a GMRS drop zone that is 1300 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft?

18.  A UH-60 is dropping 5 jumpers and 1 door bundles on a VIRS drop zone that is 275 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft?

19.  A UH-60 is dropping 2 jumpers and 2 door bundles on a VIRS drop zone that is 280 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft?

20.  A CH-47 is dropping 21 jumpers and 3 door bundles on a VIRS drop zone that is 530 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft?

21.  A C130 flying 135 KIAS is dropping 61 jumpers and 1 door bundle mass exit on a GMRS drop zone that is 1165 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft?

22.  A C17 flying 138 KIAS is dropping 91 jumpers and 2 door bundles mass exit on a GMRS drop zone that is 1450 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft?

23.  A UH-60 flying 50 KIAS is dropping 7 jumpers on a VIRS drop zone that is 295 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft?

24.  A UH-60 flying 75 KIAS is dropping 6 jumpers on a VIRS drop zone that is 600 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft?

25.  A CH-47 flying 80 KIAS is dropping 17 jumpers on a VIRS drop zone that is 415 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft?

26.  A C130 is dropping 53 jumpers and 2 door bundles mass exit on a GMRS drop zone that is 890 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft if the point of impact was moved forward 160 meters to fit the markings on the drop zone?

27.  A C17 is dropping 83 jumpers and 3 door bundles mass exit on a GMRS drop zone that is 1700 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft if the point of impact was moved forward 185 meters to fit the markings on the drop zone?

28.  A UH-60 flying 60 KIAS is dropping 4 jumpers on a VIRS drop zone that is 1350 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft if the point of impact was moved forward 95 meters to fit the markings on the drop zone?

29.  A UH-60 flying 65 KIAS is dropping 3 jumpers on a VIRS drop zone that is 430 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft if the point of impact was moved forward 65 meters to fit the markings on the drop zone?

30.  A CH-47 flying 85 KIAS is dropping 26 jumpers on a VIRS drop zone that is 540 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft if the point of impact was moved forward 100 meters to fit the markings on the drop zone?

31.  A C130 is dropping 73 jumpers and 1 door bundle mass exit on a GMRS drop zone that is 1200 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft if the point of impact was moved forward 135 meters to fit the markings on the drop zone?

32.  A C17 is dropping 94 jumpers and 1 door bundle mass exit on a GMRS drop zone that is 1850 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft if the point of impact was moved forward 112 meters to fit the markings on the drop zone?

33.  A C130 is dropping 22 jumpers and 2 door bundles mass exit on a GMRS drop zone that is 700 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft if the point of impact was moved forward 120 meters to fit the markings on the drop zone?

34.  A C17 is dropping 98 jumpers and 2 door bundles mass exit on a GMRS drop zone that is 1100 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft if the point of impact was moved forward 179 meters to fit the markings on the drop zone?

35.  A C130 is dropping 78 jumpers and 2 door bundles mass exit on a GMRS drop zone that is 2200 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft if the point of impact was moved forward 140 meters to fit the markings on the drop zone?

36.  A C17 flying 135 KIAS is dropping 89 jumpers and 3 door bundles mass exit on a GMRS drop zone that is 1600 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft if the point of impact was moved forward 107 meters to fit the markings on the drop zone?

37.  A C130 flying 135 KIAS is dropping 67 jumpers and 2 door bundles mass exit on a GMRS drop zone that is 1430 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft if the point of impact was moved forward 315 meters to fit the markings on the drop zone?

38.  A C17 flying 135 KIAS is dropping 45 jumpers and 8 door bundles mass exit on a GMRS drop zone that is 1440 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft if the point of impact was moved forward 212 meters to fit the markings on the drop zone?

39.  A C130 flying 135 KIAS is dropping 72 jumpers and 1door bundles mass exit on a GMRS drop zone that is 1100 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft if the point of impact was moved forward 155 meters to fit the markings on the drop zone?

40.  A C17 flying 135 KIAS is dropping 88 jumpers and 2 door bundles mass exit on a GMRS drop zone that is 1400 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft if the point of impact was moved forward 205 meters to fit the markings on the drop zone?

41.  A C130 is dropping 71 jumpers and 5 door bundles mass exit on a GMRS drop zone that is 1078 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft if the point of impact was moved forward 108 meters to fit the markings on the drop zone?

42.  A C17 is dropping 76 jumpers and 4 door bundles mass exit on a GMRS drop zone that is 2100 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft if the point of impact was moved forward 105 meters to fit the markings on the drop zone?

43.  A C130 is dropping 75 jumpers and 2 door bundles mass exit on a GMRS drop zone that is 1650 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft if the point of impact was moved forward 128 meters to fit the markings on the drop zone?

44.  A C17 is dropping 98 jumpers and 1 door bundle mass exit on a GMRS drop zone that is 1200 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft if the point of impact was moved forward 68 meters to fit the markings on the drop zone?

45.  A C130 is dropping 61 jumpers and 3 door bundles mass exit on a GMRS drop zone that is 1100 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft if the point of impact was moved forward 160 meters to fit the markings on the drop zone?

46.  A C17 flying 135 KIAS is dropping 89 jumpers and 2 door bundles mass exit on a GMRS drop zone that is 1760 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft if the point of impact was moved forward 320 meters to fit the markings on the drop zone?

47.  A C130 flying 135 KIAS is dropping 69 jumpers and 2 door bundles mass exit on a GMRS drop zone that is 1340 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft if the point of impact was moved forward 95 meters to fit the markings on the drop zone?

48.  A C17 flying 135 KIAS is dropping 96 jumpers and 3 door bundles mass exit on a GMRS drop zone that is 1280 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft if the point of impact was moved forward 179 meters to fit the markings on the drop zone?

49.  A C130 flying 135 KIAS is dropping 45 jumpers and 2 door bundles mass exit on a GMRS drop zone that is 1400 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft if the point of impact was moved forward 193 meters to fit the markings on the drop zone?

50.  A C17 flying 135 KIAS is dropping 92 jumpers and 5 door bundles mass exit on a GMRS drop zone that is 1765 meters long. How much time does the aircraft have to exit loads over the drop zone per pass and how many passes will it take to empty the aircraft if the point of impact was moved forward 117 meters to fit the markings on the drop zone?

|  | D=K*A*V |  |  | D=R*T |  |  | T=D/R |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1) | 54 | Meters of Drift | 1) | 1,792 | Meters Long | 1) | 13 | Seconds | 2 | Passes |
| 2) | 30 | Meters of Drift | 2) | 1,991 | Meters Long | 2) | 10 | Seconds | 3 | Passes |
| 3) | 495 | Meters of Drift | 3) | 379 | Meters Long | 3) | 2 | Seconds | 2 | Passes |
| 4) | 450 | Meters of Drift | 4) | 450 | Meters Long | 4) | 4 | Seconds | 2 | Passes |
| 5) | 48 | Meters of Drift | 5) | 1,394 | Meters Long | 5) | 13 | Seconds | 2 | Passes |
| 6) | 9 | Meters of Drift | 6) | 1,791 | Meters Long | 6) | 12 | Seconds | 2 | Passes |
| 7) | 81 | Meters of Drift | 7) | 2,586 | Meters Long | 7) | 13 | Seconds | 3 | Passes |
| 8) | 54 | Meters of Drift | 8) | 322 | Meters Long | 8) | 9 | Seconds | 1 | Passes |
| 9) | 210 | Meters of Drift | 9) | 429 | Meters Long | 9) | 9 | Seconds | 2 | Passes |
| 10) | 138 | Meters of Drift | 10) | 1,653 | Meters Long | 10) | 10 | Seconds | 4 | Passes |
| 11) | 59 | Meters of Drift | 11) | 2,587 | Meters Long | 11) | 11 | Seconds | 4 | Passes |
| 12) | 54 | Meters of Drift | 12) | 3,714 | Meters Long | 12) | 14 | Seconds | 4 | Passes |
| 13) | 25 | Meters of Drift | 13) | 450 | Meters Long | 13) | 5 | Seconds | 2 | Passes |
| 14) | 210 | Meters of Drift | 14) | 522 | Meters Long | 14) | 6 | Seconds | 2 | Passes |
| 15) | 182 | Meters of Drift | 15) | 1,486 | Meters Long | 15) | 11 | Seconds | 3 | Passes |
| 16) | 360 | Meters of Drift | 16) | 2,521 | Meters Long | 16) | 13 | Seconds | 3 | Passes |
| 17) | 270 | Meters of Drift | 17) | 3,648 | Meters Long | 17) | 16 | Seconds | 4 | Passes |
| 18) | 48 | Meters of Drift | 18) | 379 | Meters Long | 18) | 2 | Seconds | 2 | Passes |
| 19) | 863 | Meters of Drift | 19) | 379 | Meters Long | 19) | 2 | Seconds | 2 | Passes |
| 20) | 264 | Meters of Drift | 20) | 1,440 | Meters Long | 20) | 7 | Seconds | 4 | Passes |
| 21) | 660 | Meters of Drift | 21) | 2,335 | Meters Long | 21) | 13 | Seconds | 3 | Passes |
| 22) | 314 | Meters of Drift | 22) | 3,649 | Meters Long | 22) | 17 | Seconds | 3 | Passes |
| 23) | 195 | Meters of Drift | 23) | 353 | Meters Long | 23) | 3 | Seconds | 2 | Passes |
| 24) | 281 | Meters of Drift | 24) | 392 | Meters Long | 24) | 10 | Seconds | 1 | Passes |
| 25) | 330 | Meters of Drift | 25) | 853 | Meters Long | 25) | 5 | Seconds | 3 | Passes |
| 26) | 405 | Meters of Drift | 26) | 2,349 | Meters Long | 26) | 7 | Seconds | 4 | Passes |
| 27) | 315 | Meters of Drift | 27) | 3,369 | Meters Long | 27) | 19 | Seconds | 3 | Passes |
| 28) | 825 | Meters of Drift | 28) | 387 | Meters Long | 28) | 34 | Seconds | 1 | Passes |
| 29) | 621 | Meters of Drift | 29) | 332 | Meters Long | 29) | 4 | Seconds | 1 | Passes |
| 30) | 80 | Meters of Drift | 30) | 1,384 | Meters Long | 30) | 5 | Seconds | 5 | Passes |
| 31) | 479 | Meters of Drift | 31) | 2,789 | Meters Long | 31) | 12 | Seconds | 3 | Passes |
| 32) | 396 | Meters of Drift | 32) | 3,429 | Meters Long | 32) | 22 | Seconds | 3 | Passes |
| 33) | 144 | Meters of Drift | 33) | 1,249 | Meters Long | 33) | 5 | Seconds | 3 | Passes |
| 34) | 375 | Meters of Drift | 34) | 3,827 | Meters Long | 34) | 10 | Seconds | 5 | Passes |
| 35) | 263 | Meters of Drift | 35) | 3,125 | Meters Long | 35) | 27 | Seconds | 2 | Passes |
| 36) | 189 | Meters of Drift | 36) | 3,612 | Meters Long | 36) | 18 | Seconds | 3 | Passes |
| 37) | 156 | Meters of Drift | 37) | 3,063 | Meters Long | 37) | 13 | Seconds | 3 | Passes |
| 38) | 957 | Meters of Drift | 38) | 2,822 | Meters Long | 38) | 14 | Seconds | 3 | Passes |
| 39) | 273 | Meters of Drift | 39) | 2,834 | Meters Long | 39) | 10 | Seconds | 4 | Passes |
| 40) | 243 | Meters of Drift | 40) | 3,641 | Meters Long | 40) | 14 | Seconds | 4 | Passes |
| 41) | 36 | Meters of Drift | 41) | 3,093 | Meters Long | 41) | 11 | Seconds | 4 | Passes |
| 42) | 507 | Meters of Drift | 42) | 3,223 | Meters Long | 42) | 26 | Seconds | 2 | Passes |
| 43) | 1173 | Meters of Drift | 43) | 3,047 | Meters Long | 43) | 19 | Seconds | 3 | Passes |
| 44) | 144 | Meters of Drift | 44) | 3,517 | Meters Long | 44) | 13 | Seconds | 4 | Passes |
| 45) | 120 | Meters of Drift | 45) | 2,615 | Meters Long | 45) | 11 | Seconds | 3 | Passes |
| 46) | 675 | Meters of Drift | 46) | 3,825 | Meters Long | 46) | 17 | Seconds | 3 | Passes |
| 47) | 285 | Meters of Drift | 47) | 2,912 | Meters Long | 47) | 15 | Seconds | 3 | Passes |
| 48) | 429 | Meters of Drift | 48) | 3,891 | Meters Long | 48) | 13 | Seconds | 4 | Passes |
| 49) | 666 | Meters of Drift | 49) | 2,184 | Meters Long | 49) | 14 | Seconds | 2 | Passes |
| 50) | 1530 | Meters of Drift | 50) | 3,898 | Meters Long | 50) | 20 | Seconds | 3 | Passes |

CIRCULAR/RANDOM APPROACH DROP ZONES

- What radius mission specific circular CARP drop zone is required for a C130J dropping 34 jumpers and 1 door bundle mass exit at night from 1500 feet AGL using GPS?

1127 YDS

- What radius mission specific circular CARP drop zone is required for 2 C130Hs if the first one is dropping 2 HE and the second aircraft is dropping 6 CDS on a single roller system at night flying staggered trail using SKE?

931 YDS

- What radius mission specific circular CARP drop zone is required for 3 C17s dropping 101 jumpers and 2 door bundles each at night mass exit on surveyed PI using GPS?

2435 YDS

- What radius mission specific circular CARP drop zone is required for 2 C17s if the first is dropping 15 rows of HV-CDS and the second aircraft is dropping 5 HE on a dual row delivery system at night using GPS flying NIT?

1735 YDS

- What radius mission specific circular CARP drop zone is required for 1 C17 dropping 10 HAARS containers from 7500 feet AGL at night using GPS?

1250 YDS

- What radius surveyed circular CARP drop zone is required for 3 C17s dropping 67 jumpers and 2 door bundles each mass exit at night from 1800 feet AGL using GPS on off-set PIS?

3234 YDS

- What radius surveyed circular CARP drop zone is required for 2 C130Js if the lead aircraft is dropping 12 CDS on a single roller and the trail aircraft is dropping 3 HE at night?

1687 YDS

- What radius surveyed circular CARP drop zone is required for CARP DROP ZONE SIZE QUESTION NUMBER 29?

2237 YDS

- What radius surveyed circular CARP drop zone is required for CARP DROP ZONE SIZE QUESTION NUMBER 43?

3001 YDS

- What radius surveyed circular CARP drop zone is required for CARP DROP ZONE SIZE QUESTION NUMBER 54?

814 YDS