TDP Sizes

| Size | Type of AC: | Diameter of TDP | Distance between TDP’s |
| --- | --- | --- | --- |
| 1 | MH-6 / AH-6 | 10 Meters | 15 Meters |
| 2 | UH-72A / OH-58D | 15 Meters | 22.5 Meters |
| 3 | AH-1W /AH-1Z / AH-64 / UH-1Y / UH-1N | 20 Meters | 30 Meters |
| 4 | UH-60A / UH-60L / UH-60M / SH-60 | 25 Meters | 37.5 Meters |
| 5 | MV-22B / CV-22B | 30 Meters | 45 Meters |
| 6 | CH-47(D/F) / CH-53(E/K) | 35 Meters | 52.5 Meters |
| 7 | Desert/Snow Landing Zones | 100 Meters |  |
|  | Sling Load Aircraft(Day) / Unknown | 100 Meters |  |
| 8 | Sling Load long lines | 125 Meters |  |
| 9 | Sling Load Aircraft (Night) | 150 Meters |  |

| Type of AC | UH-1 (N/Y) | CH-53 | OH-58D | MH-6 AH-6 | MV-22 CV-22 | UH-60 (A/L/M) | CH-47(D/F) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Max Slope | 6 | 6 | 8 | 8 | 9 | 15 | 15 |

| A/C | UH-1N | UH-60 (A/L/M) | CH-53 | MH-6 AH-6 | MV-22 CV-22 | OH-58 | UH-1Y | CH-46/47 | AH-64 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CROSSWIND | 15 | 15 | 15 | 15 | 30 | 35 | 40 | 45 | 45 |
| TAILWIND | 15 | 15 | 15 | 15 | 10 | 20 | 40 | 10 | 45 |

Slope

Cross Wind/ Tail Wind in KNOTS

| A/C Type | Number of Emergency Exits | Location |
| --- | --- | --- |

| OH-58 | 2 | Crew doors |
| --- | --- | --- |
| UH-1 | 4 | Pilot doors and troop doors |
| UH-60 | 6 | cockpit doors and cabin windows |
| MV-22B | 6 | 6 Crew door, Ramp, both pilot windows and 2 blow out windows |
| CH-47 | 11 | 3 Primary (Ramp/Doors) 8 Secondary (Windows) |

A/C Emergency Exits

| Air Craft | Cruising Speed | Max Speed |
| --- | --- | --- |
| UH-1N | 110 KIAS | 120 KIAS |
| UH-1Y | 158 KIAS | 164 KIAS |
| UH-60L/M | 150 KIAS | 159 KIAS |
| SH-60 | 146 KIAS | 180 KIAS |
| CH-47D/F | 130 KIAS | 170 KIAS |
| CH-53E | 150 KIAS | 200 KIAS |
| CH-53K | 160 KIAS | 200 KIAS |
| V-22 | 220 KIAS | 250 KIAS |

A/C Air Speeds