REVISED 14 FEBRUARY 2011

WTC PATHFINDER SLING LOAD DEFICIENCIES

HMMWV						Cargo Net/Fuel Drum

MAJOR						         MAJOR

25.  Incorrect Fuel Level				   25.  Hooks Not Taped

26.  Parking Brake Not Engaged			   26.  Wrong POA to Load

27.  Incorrect Wheel Alignment			    27.  Clevis Not Taped

MINOR							MINOR

28.  Hood Latch Not Secured				  28.  Apex Tether Cord Missing

29.  Hood Latch Not Taped				  29.  Apex Tether Not on #1 Hook

30.  Mirrors Not Secured				  30.  Incorrect Suspension Leg Sequence

31.  Mirrors Not Facing Forward			  31.  Hooks Not Alternating

32.  Steering Wheel Not Secured			  32.  Quick Disconnect Adapter Not Taped

33.  Transmission Not in Neutral			  33.  5/8” Screw Pin Clevis Missing Cotter Pin

34.  Glass/Plastic Not Shatter-proofed		  34.  Clevis Not Hand Tight

35.  Troop Seats Not Secured	                                    35.  A7A Excess Secured Over Friction Adapter

36.  Seat Belt Not Secured			             36.  A7A Excess Not Secured

37.  Fuel Cap Not Secured			             37.  A7A Excess Incorrectly Secured

38.  Key Missing from Lock			             38.  A7A Misrouted

39.  Hooks Not Facing Out			             39.  Suspension Leg Out of Break-Away

40.  Hooks Not Secured       				  40.  Suspension Leg Misrouted Thru Net

41.  Engine Switch Not in Off Position                       41.  Load Not 75% Centered

42.  Lights Not in Off Position                                      42.  Hole in Top of Load Larger that Smallest

43.  Battery Box Latch Not Secured                                   Object

44.  Battery Box Latch Not Taped			  43.  A7A Misrouted thru Friction Adapter

REVISED 14 FEBRUARY 2011

WTC PATHFINDER SLING LOAD DEFICIENCIES

WATER TRAILER					A22 CARGO BAG

MAJOR						        MAJOR

25.  Buffer Missing				           25.  Clevis Not Taped

26.  Parking Brake Not Engaged		           26.  Incorrect POA to Load

27.  Chains Not Mirrored thru Lanett                      27.  Incorrect 24” Suspension Web Sequence

MINOR

MINOR

28.  Fill Port Not Secured			           28.  Butterfly Snap Hook Facing Up

29.  Hoses Not Secured 			           29.  Butterfly Snap Hook Not Taped

30.  Faucet Box Not Secured			           30.  Lacing Cord Misrouted

31.  Jack Handle Not Secured  		           31.  Incorrect Lacing Cord

32.  Safety Chains Not Secured		           32.  Lacing Cord Missing

33.  Glass/Plastic Not Shatter-proofed	           33.  Excess Lacing Cord Not Secured

34.  Chains Not Mirrored in Rear		           34.  24” Suspension Web Twisted

35.  188”/Lateral Strap Misrouted

36.  188”/Lateral Strap Excess Not Secured

37.  188”/Lateral Strap Excess Incorrectly Rolled

38.  188”/Lateral Strap Excess Secured Over

Friction Adapter

39.  Clevis Not Hand Tight

40.  188”/Lateral Strap has Quick Release

41.  188”/Lateral Misrouted thru Friction Adapter

42. 188”/Lateral Twisted

REVISED 14 FEBRUARY 2011

WTC PATHFINDER SLING LOAD DEFICIENCIES

COMMON TO ALL

MAJOR

- Castellated Nut Missing

- Snap Ring Missing

- Grab Hook Assembly Inverted

- Lock/Dome Nut Missing

- Weight of Load Incorrect

6.  Weight of Load Exceeds A/C Cargo Hook Capacity

7.  F.O.D.

8.  Sling Leg Misrouted

9.  Chains Misrouted

10.  Incorrect Link Count

11.  Weight Exceeds Sling Leg Capacity

12.  Incorrect Apex on Sling Set

13.  Incorrect POA to A/C

14.  Cargo Hook Reach Pendant Inverted

15.  10K Apex Missing Spacer

16.  Sling Leg Twisted

17.  Permanent Tie v/s Break-Away

MINOR

18.  Castellated Nut Inverted

19.  Incorrect Spacer on Apex

20.  Cotter Pin Missing from Apex

21.  Excess Improperly Secured (Ramp or not secured to itself)

22.  Buffer Missing from Donut Ring

23.  Missing Break-Away

24.  Type IV Connector Link Missing Aluminum Buffer

REVISED 14 FEBRUARY 2011

WTC PATHFINDER SLING LOAD DEFICIENCIES

HIGH MOBILITY TRAILER

MAJOR

25.  	Parking Brake Not Engaged

26.  	Load Incorrectly Secured

27.   Buffer Missing

MINOR

28.  Light Cable Not Secured

29.  Tow Hooks Not Secured

30.  Rear Jacks Not Secured

31.  Tailgate Not Secured

32.  Glass/Plastic Not Shatter-proofed

33.  Rear Jacks Not Fully Extended

MAJOR= -32 PTS

MINOR= -16 PTS